{

"schema": "1.0",

"title": "CLASS D FIRE — FULL BRANCHING ADVENTURE TREE",

"category": "Class D Fire (Combustible Metals: Magnesium, Titanium, Sodium, Lithium, Aluminum Dust)",

"root": "D1",

"phases": [

{ "id": 1, "name": "Core Combustible Metal Tree" },

{ "id": 2, "name": "Catastrophic Combustible Metal Events" }

],

"nodes": {

"D1": {

"phase": 1,

"title": "NODE D1 — Initial Indicator",

"situation": "You notice an intense white flame and sparks coming from a workshop area. Someone shouts, “Metal’s burning!” The fire produces blinding light and extreme heat. No alarm is sounding yet.",

"question": "What do you do first?",

"options": [

{ "label": "A", "text": "Pull the fire alarm immediately", "outcome": { "type": "node", "target": "D2" } },

{ "label": "B", "text": "Throw water on it", "outcome": { "type": "failure", "target": "FD1" } },

{ "label": "C", "text": "Investigate closer to confirm", "outcome": { "type": "failure", "target": "FD2" } },

{ "label": "D", "text": "Attempt to smother it with a cloth", "outcome": { "type": "failure", "target": "FD3" } }

]

},

"D2": {

"phase": 1,

"title": "NODE D2 — Alarm Activated",

"situation": "The alarm sounds. People evacuate, but some stop to stare at the bright flames.",

"question": "What is your priority?",

"options": [

{ "label": "A", "text": "Force evacuation away from the area", "outcome": { "type": "node", "target": "D3" } },

{ "label": "B", "text": "Allow people to watch briefly", "outcome": { "type": "failure", "target": "FD4" } },

{ "label": "C", "text": "Attempt suppression immediately", "outcome": { "type": "node", "target": "D4" } },

{ "label": "D", "text": "Silence alarm to reduce panic", "outcome": { "type": "failure", "target": "FD5" } }

]

},

"D3": {

"phase": 1,

"title": "NODE D3 — Fire Characteristics",

"situation": "The fire is extremely bright, emitting sparks and molten droplets. Smoke is minimal, but heat is intense.",

"question": "What does this indicate?",

"options": [

{ "label": "A", "text": "Class D metal fire", "outcome": { "type": "node", "target": "D4" } },

{ "label": "B", "text": "Class A fire", "outcome": { "type": "failure", "target": "FD6" } },

{ "label": "C", "text": "Class B fire", "outcome": { "type": "failure", "target": "FD6" } },

{ "label": "D", "text": "Electrical fire", "outcome": { "type": "failure", "target": "FD6" } }

]

},

"D4": {

"phase": 1,

"title": "NODE D4 — Suppression Decision",

"situation": "A fire extinguisher cabinet is nearby.",

"question": "Which extinguisher is appropriate?",

"options": [

{ "label": "A", "text": "Water", "outcome": { "type": "failure", "target": "FD1" } },

{ "label": "B", "text": "CO₂", "outcome": { "type": "failure", "target": "FD7" } },

{ "label": "C", "text": "Foam", "outcome": { "type": "failure", "target": "FD7" } },

{ "label": "D", "text": "Class D dry powder", "outcome": { "type": "node", "target": "D5" } }

]

},

"D5": {

"phase": 1,

"title": "NODE D5 — Class D Suppression Attempt",

"situation": "You apply dry powder. The metal continues burning beneath the surface.",

"question": "What is the correct technique?",

"options": [

{ "label": "A", "text": "Smother completely and allow to cool", "outcome": { "type": "node", "target": "D6" } },

{ "label": "B", "text": "Stir the metal to expose flames", "outcome": { "type": "failure", "target": "FD8" } },

{ "label": "C", "text": "Add water to cool faster", "outcome": { "type": "failure", "target": "FD1" } },

{ "label": "D", "text": "Apply powder briefly and leave", "outcome": { "type": "failure", "target": "FD9" } }

]

},

"D6": {

"phase": 1,

"title": "NODE D6 — Cooling Period",

"situation": "Flames subside, but the metal remains extremely hot and glowing.",

"question": "What must NOT happen?",

"options": [

{ "label": "A", "text": "Disturbing the metal", "outcome": { "type": "failure", "target": "FD10" } },

{ "label": "B", "text": "Continued isolation", "outcome": { "type": "node", "target": "D7" } },

{ "label": "C", "text": "Extended cooling time", "outcome": { "type": "node", "target": "D7" } },

{ "label": "D", "text": "Monitoring for re-ignition", "outcome": { "type": "node", "target": "D7" } }

]

},

"D7": {

"phase": 1,

"title": "NODE D7 — Re-Ignition Risk",

"situation": "Someone suggests moving the metal outside “to be safe.”",

"question": "What do you do?",

"options": [

{ "label": "A", "text": "Prevent movement and maintain isolation", "outcome": { "type": "node", "target": "D8" } },

{ "label": "B", "text": "Allow trained staff to move it", "outcome": { "type": "failure", "target": "FD11" } },

{ "label": "C", "text": "Assist with relocation", "outcome": { "type": "failure", "target": "FD12" } },

{ "label": "D", "text": "Hose it down outside", "outcome": { "type": "failure", "target": "FD1" } }

]

},

"D8": {

"phase": 1,

"title": "NODE D8 — Unknown Metal Composition",

"situation": "It is unclear which metal is burning. Sparks change color intermittently.",

"question": "What is the safest assumption?",

"options": [

{ "label": "A", "text": "Treat as worst-case Class D", "outcome": { "type": "node", "target": "D9" } },

{ "label": "B", "text": "Assume aluminum only", "outcome": { "type": "failure", "target": "FD13" } },

{ "label": "C", "text": "Assume lithium", "outcome": { "type": "failure", "target": "FD13" } },

{ "label": "D", "text": "Wait to identify before acting", "outcome": { "type": "failure", "target": "FD14" } }

]

},

"D9": {

"phase": 1,

"title": "NODE D9 — Secondary Hazard Identified",

"situation": "Fine metallic dust is present on nearby surfaces.",

"question": "What new risk exists?",

"options": [

{ "label": "A", "text": "Dust explosion", "outcome": { "type": "node", "target": "D10" } },

{ "label": "B", "text": "Reduced fire risk", "outcome": { "type": "failure", "target": "FD15" } },

{ "label": "C", "text": "Smoke inhalation only", "outcome": { "type": "failure", "target": "FD15" } },

{ "label": "D", "text": "Electrical hazard", "outcome": { "type": "failure", "target": "FD15" } }

]

},

"D10": {

"phase": 1,

"title": "NODE D10 — Dust Disturbance Risk",

"situation": "People evacuating begin kicking up dust.",

"question": "What instruction is critical?",

"options": [

{ "label": "A", "text": "Move slowly and avoid disturbing surfaces", "outcome": { "type": "node", "target": "D11" } },

{ "label": "B", "text": "Run quickly", "outcome": { "type": "failure", "target": "FD16" } },

{ "label": "C", "text": "Sweep the dust", "outcome": { "type": "failure", "target": "FD16" } },

{ "label": "D", "text": "Use fans to clear air", "outcome": { "type": "failure", "target": "FD16" } }

]

},

"D11": {

"phase": 1,

"title": "NODE D11 — Dust Ignition",

"situation": "A small ignition occurs in suspended dust.",

"question": "What must happen immediately?",

"options": [

{ "label": "A", "text": "Expand evacuation zone", "outcome": { "type": "node", "target": "D12" } },

{ "label": "B", "text": "Attempt suppression", "outcome": { "type": "failure", "target": "FD17" } },

{ "label": "C", "text": "Ventilate aggressively", "outcome": { "type": "failure", "target": "FD18" } },

{ "label": "D", "text": "Move closer to assess", "outcome": { "type": "failure", "target": "FD19" } }

]

},

"D12": {

"phase": 1,

"title": "NODE D12 — Structural Heat Transfer",

"situation": "Heat from the metal fire transfers into surrounding steel supports.",

"question": "What is the concern?",

"options": [

{ "label": "A", "text": "Structural weakening", "outcome": { "type": "node", "target": "D13" } },

{ "label": "B", "text": "Reduced fire intensity", "outcome": { "type": "failure", "target": "FD20" } },

{ "label": "C", "text": "Alarm failure", "outcome": { "type": "failure", "target": "FD20" } },

{ "label": "D", "text": "Smoke reduction", "outcome": { "type": "failure", "target": "FD20" } }

]

},

"D13": {

"phase": 1,

"title": "NODE D13 — Authority Pressure",

"situation": "A supervisor demands suppression stop to “save equipment.”",

"question": "How do you respond?",

"options": [

{ "label": "A", "text": "Deny request citing life safety", "outcome": { "type": "node", "target": "D14" } },

{ "label": "B", "text": "Comply", "outcome": { "type": "failure", "target": "FD21" } },

{ "label": "C", "text": "Delay evacuation", "outcome": { "type": "failure", "target": "FD21" } },

{ "label": "D", "text": "Silence alarm", "outcome": { "type": "failure", "target": "FD21" } }

]

},

"D14": {

"phase": 1,

"title": "NODE D14 — Extended Isolation",

"situation": "The fire is controlled but requires long cooling. People want to re-enter.",

"question": "What do you do?",

"options": [

{ "label": "A", "text": "Prevent re-entry until fully cooled", "outcome": { "type": "node", "target": "D15" } },

{ "label": "B", "text": "Allow brief access", "outcome": { "type": "failure", "target": "FD22" } },

{ "label": "C", "text": "Let individuals decide", "outcome": { "type": "failure", "target": "FD22" } },

{ "label": "D", "text": "Declare incident over", "outcome": { "type": "failure", "target": "FD22" } }

]

},

"D15": {

"phase": 1,

"title": "NODE D15 — Post-Incident Risk",

"situation": "Someone proposes cleanup before responders clear the area.",

"question": "What is the danger?",

"options": [

{ "label": "A", "text": "Re-ignition and burns", "outcome": { "type": "success", "target": "SD1" } },

{ "label": "B", "text": "No danger", "outcome": { "type": "failure", "target": "FD23" } },

{ "label": "C", "text": "Smoke irritation only", "outcome": { "type": "failure", "target": "FD23" } },

{ "label": "D", "text": "Alarm disruption", "outcome": { "type": "failure", "target": "FD23" } }

]

},

"D16": {

"phase": 2,

"title": "NODE D16 — Misidentified Metal (Critical Error)",

"situation": "Post-suppression, someone claims the metal was “probably aluminum scraps” and suggests normal cleanup.",

"question": "What is the safest assumption?",

"options": [

{ "label": "A", "text": "Treat all residue as reactive metal", "outcome": { "type": "node", "target": "D17" } },

{ "label": "B", "text": "Assume it is safe once flames stop", "outcome": { "type": "failure", "target": "FD24" } },

{ "label": "C", "text": "Spray with water to confirm", "outcome": { "type": "failure", "target": "FD25" } },

{ "label": "D", "text": "Handle with gloves and move it", "outcome": { "type": "failure", "target": "FD26" } }

]

},

"D17": {

"phase": 2,

"title": "NODE D17 — Latent Reactivity",

"situation": "The metal appears dull but emits heat waves. No flames visible.",

"question": "What invisible danger remains?",

"options": [

{ "label": "A", "text": "Delayed re-ignition", "outcome": { "type": "node", "target": "D18" } },

{ "label": "B", "text": "Smoke toxicity", "outcome": { "type": "failure", "target": "FD27" } },

{ "label": "C", "text": "Electrical hazard", "outcome": { "type": "failure", "target": "FD27" } },

{ "label": "D", "text": "Structural collapse", "outcome": { "type": "failure", "target": "FD27" } }

]

},

"D18": {

"phase": 2,

"title": "NODE D18 — Premature Cleanup Attempt",

"situation": "A worker begins sweeping metal residue into a container.",

"question": "What do you do?",

"options": [

{ "label": "A", "text": "Stop immediately and isolate", "outcome": { "type": "node", "target": "D19" } },

{ "label": "B", "text": "Allow cleanup to continue", "outcome": { "type": "failure", "target": "FD28" } },

{ "label": "C", "text": "Assist to finish faster", "outcome": { "type": "failure", "target": "FD29" } },

{ "label": "D", "text": "Add water to cool residue", "outcome": { "type": "failure", "target": "FD25" } }

]

},

"D19": {

"phase": 2,

"title": "NODE D19 — Container Ignition Risk",

"situation": "The residue container begins heating rapidly.",

"question": "What is happening?",

"options": [

{ "label": "A", "text": "Heat buildup leading to ignition", "outcome": { "type": "node", "target": "D20" } },

{ "label": "B", "text": "Normal cooling", "outcome": { "type": "failure", "target": "FD30" } },

{ "label": "C", "text": "Safe neutralization", "outcome": { "type": "failure", "target": "FD30" } },

{ "label": "D", "text": "Alarm malfunction", "outcome": { "type": "failure", "target": "FD30" } }

]

},

"D20": {

"phase": 2,

"title": "NODE D20 — Secondary Explosion",

"situation": "The container ruptures violently, scattering burning metal fragments.",

"question": "What caused this escalation?",

"options": [

{ "label": "A", "text": "Confinement of reactive metal", "outcome": { "type": "node", "target": "D21" } },

{ "label": "B", "text": "Air exposure", "outcome": { "type": "failure", "target": "FD31" } },

{ "label": "C", "text": "Alarm delay", "outcome": { "type": "failure", "target": "FD31" } },

{ "label": "D", "text": "Improper extinguisher", "outcome": { "type": "failure", "target": "FD31" } }

]

},

"D21": {

"phase": 2,

"title": "NODE D21 — Multiple Ignition Points",

"situation": "Several secondary metal fires ignite across the workshop.",

"question": "What is the FIRST priority?",

"options": [

{ "label": "A", "text": "Expand evacuation immediately", "outcome": { "type": "node", "target": "D22" } },

{ "label": "B", "text": "Attempt suppression at nearest fire", "outcome": { "type": "failure", "target": "FD32" } },

{ "label": "C", "text": "Move equipment away", "outcome": { "type": "failure", "target": "FD33" } },

{ "label": "D", "text": "Shut off power", "outcome": { "type": "failure", "target": "FD34" } }

]

},

"D22": {

"phase": 2,

"title": "NODE D22 — Metal Dust Cloud",

"situation": "Explosions disperse fine metal dust into the air.",

"question": "What extreme hazard now exists?",

"options": [

{ "label": "A", "text": "Dust explosion chain reaction", "outcome": { "type": "node", "target": "D23" } },

{ "label": "B", "text": "Reduced oxygen", "outcome": { "type": "failure", "target": "FD35" } },

{ "label": "C", "text": "Smoke inhalation only", "outcome": { "type": "failure", "target": "FD35" } },

{ "label": "D", "text": "Alarm overload", "outcome": { "type": "failure", "target": "FD35" } }

]

},

"D23": {

"phase": 2,

"title": "NODE D23 — Dust Explosion Trigger",

"situation": "A single spark flashes near the ceiling.",

"question": "What is MOST likely to occur?",

"options": [

{ "label": "A", "text": "Rapid pressure wave explosion", "outcome": { "type": "node", "target": "D24" } },

{ "label": "B", "text": "Fire extinguishment", "outcome": { "type": "failure", "target": "FD36" } },

{ "label": "C", "text": "Minor flare only", "outcome": { "type": "failure", "target": "FD36" } },

{ "label": "D", "text": "No effect", "outcome": { "type": "failure", "target": "FD36" } }

]

},

"D24": {

"phase": 2,

"title": "NODE D24 — Primary Dust Explosion",

"situation": "A massive pressure wave blows out walls and doors. A fireball rolls through the space.",

"question": "What determines survival now?",

"options": [

{ "label": "A", "text": "Distance and shielding", "outcome": { "type": "node", "target": "D25" } },

{ "label": "B", "text": "Fire extinguisher choice", "outcome": { "type": "failure", "target": "FD37" } },

{ "label": "C", "text": "Alarm timing", "outcome": { "type": "failure", "target": "FD37" } },

{ "label": "D", "text": "Authority presence", "outcome": { "type": "failure", "target": "FD37" } }

]

},

"D25": {

"phase": 2,

"title": "NODE D25 — Secondary Dust Explosion",

"situation": "The initial blast stirs settled dust elsewhere in the building.",

"question": "What is the danger?",

"options": [

{ "label": "A", "text": "Secondary explosions propagating", "outcome": { "type": "node", "target": "D26" } },

{ "label": "B", "text": "Fire reduction", "outcome": { "type": "failure", "target": "FD38" } },

{ "label": "C", "text": "Structural reinforcement", "outcome": { "type": "failure", "target": "FD38" } },

{ "label": "D", "text": "Smoke dispersal", "outcome": { "type": "failure", "target": "FD38" } }

]

},

"D26": {

"phase": 2,

"title": "NODE D26 — Structural Failure",

"situation": "Load-bearing supports weaken from heat and blast damage.",

"question": "What must happen immediately?",

"options": [

{ "label": "A", "text": "Establish collapse zone", "outcome": { "type": "node", "target": "D27" } },

{ "label": "B", "text": "Resume suppression", "outcome": { "type": "failure", "target": "FD39" } },

{ "label": "C", "text": "Re-enter to rescue property", "outcome": { "type": "failure", "target": "FD40" } },

{ "label": "D", "text": "Silence alarms", "outcome": { "type": "failure", "target": "FD41" } }

]

},

"D27": {

"phase": 2,

"title": "NODE D27 — Collapse Event",

"situation": "Part of the structure collapses inward.",

"question": "What is the primary response?",

"options": [

{ "label": "A", "text": "Account for all personnel", "outcome": { "type": "node", "target": "D28" } },

{ "label": "B", "text": "Continue suppression", "outcome": { "type": "failure", "target": "FD42" } },

{ "label": "C", "text": "Attempt rescue without PPE", "outcome": { "type": "failure", "target": "FD43" } },

{ "label": "D", "text": "Resume evacuation only", "outcome": { "type": "failure", "target": "FD44" } }

]

},

"D28": {

"phase": 2,

"title": "NODE D28 — Mass Casualty Scenario",

"situation": "Multiple injuries from burns, blast trauma, and collapse.",

"question": "What takes priority?",

"options": [

{ "label": "A", "text": "Life safety and triage", "outcome": { "type": "node", "target": "D29" } },

{ "label": "B", "text": "Fire origin investigation", "outcome": { "type": "failure", "target": "FD45" } },

{ "label": "C", "text": "Media coordination", "outcome": { "type": "failure", "target": "FD46" } },

{ "label": "D", "text": "Property recovery", "outcome": { "type": "failure", "target": "FD47" } }

]

},

"D29": {

"phase": 2,

"title": "NODE D29 — Resource Overload",

"situation": "Emergency resources are stretched thin.",

"question": "What prevents further loss of life?",

"options": [

{ "label": "A", "text": "Incident command discipline", "outcome": { "type": "node", "target": "D30" } },

{ "label": "B", "text": "Speed alone", "outcome": { "type": "failure", "target": "FD48" } },

{ "label": "C", "text": "Individual heroics", "outcome": { "type": "failure", "target": "FD48" } },

{ "label": "D", "text": "Silencing alarms", "outcome": { "type": "failure", "target": "FD48" } }

]

},

"D30": {

"phase": 2,

"title": "NODE D30 — Stabilization Phase",

"situation": "Fires are contained. Dust has settled. Collapse risk remains.",

"question": "What phase is the incident in?",

"options": [

{ "label": "A", "text": "Stabilization", "outcome": { "type": "node", "target": "D31" } },

{ "label": "B", "text": "Resolution", "outcome": { "type": "failure", "target": "FD49" } },

{ "label": "C", "text": "Normal operations", "outcome": { "type": "failure", "target": "FD49" } },

{ "label": "D", "text": "Investigation only", "outcome": { "type": "failure", "target": "FD49" } }

]

},

"D31": {

"phase": 2,

"title": "NODE D31 — Final Preventable Mistake",

"situation": "Someone suggests entering to retrieve equipment before clearance.",

"question": "What do you do?",

"options": [

{ "label": "A", "text": "Deny entry until fully cleared", "outcome": { "type": "success", "target": "SD2" } },

{ "label": "B", "text": "Allow quick retrieval", "outcome": { "type": "failure", "target": "FD50" } },

{ "label": "C", "text": "Escort them inside", "outcome": { "type": "failure", "target": "FD50" } },

{ "label": "D", "text": "Ignore request", "outcome": { "type": "failure", "target": "FD50" } }

]

}

},

"end_states": {

"success": {

"SD1": {

"label": "Success SD1 — Metal Fire Controlled",

"description": "Correct recognition, specialized suppression, dust hazard management, extended cooling, no injuries."

},

"SD2": {

"label": "Success SD2 — Worst-Case Survival",

"description": "Catastrophic metal fire and dust explosion occurred, but evacuation, isolation, and command discipline prevented mass fatalities."

}

},

"failure_ranges": {

"FD1_FD23": "Water reaction explosions, dust explosions, re-ignition burns, structural compromise, authority override failures.",

"FD24_FD50": "Reactivity mishandling, dust explosions, structural collapse deaths, secondary ignition cascades, command failure."

}

}

}